Section 1: Production Context

This is the most important section because this is where the meaning comes in.

Fields:

Production Run ID

Start Time

End Time

Line / Extruder Number

Product Name

Product Code / ERP Article Number

Customer / Order

Batch / Lot Number

Recipe ID

Material Type

Material Grade

Supplier

Virgin / Recycled / Mixed

Recycled Percentage

Silo Path / Silo Route

Input Source

Co-Extruder active yes/no

Operator

Shift

Notes

Especially important for you:

Silo path

which material was introduced

what is currently being produced

whether it contains recycled material

which mixture is being used

This is not optional.

This will be invaluable for AI later.

Section 2: Process Parameters / Recipe Snapshot

This is where you enter what can be retrieved from the production sheet or later from ERP/machine.

From your displayed sheet, the following are relevant: B. Relevant:

Extruder zone temperatures (setpoint + actual)

Co-extruder temperatures

Tool/die zone temperatures

Adapter temperature

Screw speed

Dosing speed

Motor load

Melt pressure

Melt temperature

Line speed

Water settings

Vacuum settings

Pump status/pump level

Puller/calibrator related values

Water temperature

Vacuum tank values

Pressure/counter-pressure

Output/throughput kg/h

Dimension/profile related values, if available

Very important: Don't just save actual values, but also, where possible:

Setpoint

Actual

Deviation

Section 3: Quality/Outcome

This is often missing, but crucial for quality loss.

This is where the following should go:

Quality status: OK / Warning / NOK

Scrap amount

Scrap percentage

Defect type

Defect description

Visual defect yes/no

Dimensional issue yes/no

Surface issue yes/no

Color deviation yes/no

Density / weight deviation yes/no

Customer complaint reference

Internal QC result

Lab result / QA result

Rework yes/no

Downgrade yes/no

Shift issue yes/no

Changeover issue yes/no

Stop/start instability yes/no

Notes

Without an outcome, the AI ​​can only say later: “The print was different.”

With an outcome, it can say: “This pattern often led to a loss of quality.”

That's the huge difference.

Section 4: AI Detected Profile

I would display this section separately, not maintain it manually.

Here it says later:

Detected profile ID

Confidence

Closest historical profile

Baseline used

Material variant inferred

Drift score

Quality risk score

Abnormal score

Transition detected yes/no

Similar past runs

Explanation text

And then optionally:

“Assign label”

“Confirm detected material”

“Correct detected profile”