Yes. The whole system must be built so the interface does not merely show nice dummy values, but already works today with the existing Zitta data—and later expands automatically as soon as new data sources are connected.

The most important point: you must not mix three different metrics.

**Digitalization progress** shows how much infrastructure and how many data sources are already connected.

**Data quality score** shows how complete, current, and reliable the incoming data are.

**Model performance** shows how well an actually trained and validated model makes predictions.

As long as there are no real validated predictions, the system should not show “Accuracy 71 %.” That would be technically misleading. Instead it should show “Prediction readiness 71 %” or “AI Readiness 71 %.” A real accuracy may only be shown once the model has been evaluated against real events and historical data.

---

**1. What you already have at Zitta**

Based on our current understanding, you already have the following foundation:

- An edge server or AI server

- Access to production or machine data

- SQL data from the plant

- Live signals or sensor values

- Machine states such as OFF, READY, HEATING UP, PRODUCTION, and COOLING DOWN

- Basic process signals such as temperature, pressure, speed, motor load, and possibly throughput

- A web dashboard

- Sensor management

- Historical runs, or at least the technical structure for them

- Machines and material profiles

- An initial baseline logic or rules for process assessment

What is currently likely still missing: reliable quality data, maintenance data, material batches, real fault labels, scrap data, energy consumption, complete production orders, and robust links between events and process data.

That is exactly why the system can already monitor, but cannot yet make reliable predictions in every area.

---

**2. The backend must be built in a data-source-oriented way**

The developer should not hard-code every new feature. Instead you need a central data-source and capability logic.

The system must know:

- Which data source is available?

- Which fields does it provide?

- How current are the data?

- How complete are the data?

- Which modules need these data?

- Which features are already usable as a result?

- Which features remain locked?

- Which additional data source would bring the greatest added value?

For that you need a central table or collection named, for example:

`data_sources`

Example structure:

```json

{

"id": "source_machine_sql",

"name": "Machine SQL Database",

"category": "machine_process",

"status": "connected",

"connection_type": "mssql",

"machine_id": "extruder_01",

"last_seen_at": "2026-07-22T14:30:00Z",

"expected_interval_seconds": 10,

"completeness_score": 0.92,

"freshness_score": 0.98,

"reliability_score": 0.95,

"validated": true

}

```

Further data sources could be:

- `machine_process`

- `machine_state`

- `quality`

- `maintenance`

- `material`

- `production_order`

- `energy`

- `operator_events`

- `alarm_history`

- `laboratory`

- `camera`

- `erp`

---

**3. Every feature needs defined prerequisites**

For every feature, a definition must be stored in the backend.

Example:

`feature_capabilities`

One feature would be, for example, “Predict quality degradation.”

```json

{

"feature_key": "quality_degradation_prediction",

"name": "Quality degradation",

"required_sources": [

"machine_process",

"machine_state"

],

"recommended_sources": [

"quality",

"material",

"operator_events"

],

"validation_required": true,

"minimum_history_days": 30

}

```

Another module:

```json

{

"feature_key": "remaining_useful_life",

"name": "Remaining tool life",

"required_sources": [

"machine_process",

"maintenance"

],

"recommended_sources": [

"alarm_history",

"operator_events"

],

"validation_required": true,

"minimum_history_days": 60

}

```

One more example:

```json

{

"feature_key": "energy_optimization",

"name": "Energy optimization",

"required_sources": [

"machine_process",

"energy"

],

"recommended_sources": [

"production_order",

"material"

]

}

```

This allows the backend to decide automatically:

- Available

- Partially available

- In setup

- Locked

- Not yet validated

---

**4. Status model for every feature**

Every feature should have one of these statuses:

- `LOCKED`

- `SETUP_REQUIRED`

- `COLLECTING_DATA`

- `TRAINING`

- `VALIDATION_REQUIRED`

- `ACTIVE`

- `DEGRADED`

- `ERROR`

Example:

Quality prediction is `LOCKED` as long as no quality data are connected.

After connection it becomes `COLLECTING_DATA`.

After enough historical data it becomes `TRAINING`.

After model training it becomes `VALIDATION_REQUIRED`.

Only after validated review does it become `ACTIVE`.

This is important because it keeps the interface honest while still showing the target vision.

---

**5. Calculating digitalization progress**

Progress must not simply be entered manually. It should be calculated from actual completed technical steps.

Example weighting:

- AI server reachable: 10 %

- Production data connected: 15 %

- Machine status available: 10 %

- One machine fully integrated: 10 %

- Production history available: 10 %

- Quality data connected: 15 %

- Maintenance data connected: 10 %

- Material batches connected: 5 %

- Energy data connected: 5 %

- Operator events available: 5 %

- Models validated: 5 %

Calculation:

`progress = sum of all fulfilled weights`

Example:

- Server available: 10 %

- Production data available: 15 %

- Machine status available: 10 %

- One machine connected: 10 %

- History partially available: 5 of 10 %

Total: **50 %**

The values should not come from the UI, but from the backend.

---

**6. Calculate machine progress separately**

Because Zitta has many machines, each machine needs its own integration status.

Table:

`machine_integrations`

Example:

```json

{

"machine_id": "extruder_01",

"machine_name": "Extruder 1",

"network_connected": true,

"process_data_connected": true,

"state_data_connected": true,

"quality_linked": false,

"maintenance_linked": false,

"material_linked": false,

"energy_linked": false,

"integration_score": 42

}

```

A machine that is not connected, for example:

```json

{

"machine_id": "extruder_02",

"network_connected": false,

"process_data_connected": false,

"integration_score": 0

}

```

In the dashboard it can then say:

**1 of 20 machines connected**

That is clearer for Zitta than an abstract technical statement.

---

**7. Difference between progress and prediction readiness**

Progress shows how far the overall project is.

Prediction readiness shows how well the system could learn from today’s perspective.

Example calculation:

**Prediction Readiness =**

30 % data coverage

20 % data quality

20 % history length

15 % label coverage

10 % machine coverage

5 % system stability

Example:

- Data coverage: 80 %

- Data quality: 90 %

- History length: 40 %

- Labels: 10 %

- Machine coverage: 5 %

- System stability: 95 %

Weighted, that might yield **48 %**.

That does not mean the predictions are 48 % correct. It means the foundation for good models is currently 48 % in place.

---

**8. Store real model performance separately**

Once a model is actually trained, you need a table:

`model_versions`

Example:

```json

{

"model_id": "quality_risk_extruder_01",

"version": "1.3.0",

"target": "quality_degradation",

"machine_id": "extruder_01",

"trained_from": "2026-01-01",

"trained_to": "2026-06-30",

"sample_count": 152340,

"positive_events": 84,

"precision": 0.81,

"recall": 0.74,

"f1_score": 0.77,

"false_positive_rate": 0.09,

"validation_status": "approved",

"deployed": true

}

```

In the dashboard it should not simply say “Accuracy.” Better, depending on the model:

- Detection rate

- Hit rate / precision

- False alarm rate

- Lead time

- Validation status

Example:

**Quality risk model**

Precision: 81 %

Recall: 74 %

False alarm rate: 9 %

Average lead time: 43 minutes

That is clearly more professional.

---

**9. Using dummy data correctly**

For the target vision you can use dummy data, but they must be technically marked as simulation.

In the database, every result should have a field:

```json

{

"value_source": "SIMULATED"

}

```

Possible values:

- `LIVE`

- `DERIVED`

- `RULE_BASED`

- `MODEL_PREDICTION`

- `SIMULATED`

- `MANUAL`

Example:

```json

{

"prediction_type": "bearing_failure",

"probability": 0.72,

"value_source": "SIMULATED",

"display_label": "Example visualization",

"is_customer_decision_relevant": false

}

```

In a customer demo, a small note can appear at the top:

**Demo mode – simulated predictions**

That way the effect can be strong without making technically false claims.

---

**10. How features can already be visible today**

Even a feature that is not yet active can be fully designed.

Example:

**Quality prediction**

Status: Not yet activated

Required for activation:

1. Connect quality data source

2. Link quality value to production timestamp

3. Collect at least 30 days of data

4. Validate model

The user can click “Start connection.”

Then a setup wizard opens.

---

**11. Setup wizard for new data sources**

Every missing data source should be connected via a guided process.

Example quality data:

**Step 1:** Select source.

SQL. Excel. CSV. API. Manual entry. Lab software.

**Step 2:** Map fields.

Production run. Machine. Timestamp. Material batch. Quality value. Approval status. Scrap.

**Step 3:** Data preview.

**Step 4:** Check data quality.

**Step 5:** Import historical data.

**Step 6:** Activate feature.

---

**12. Data model in the backend**

At least the following main tables or collections should exist:

- `companies`

- `sites`

- `production_lines`

- `machines`

- `machine_components`

- `sensors`

- `sensor_mappings`

- `sensor_readings`

- `machine_states`

- `production_runs`

- `materials`

- `material_batches`

- `quality_results`

- `maintenance_events`

- `alarms`

- `operator_events`

- `data_sources`

- `data_source_health`

- `feature_capabilities`

- `feature_status`

- `integration_progress`

- `model_versions`

- `model_predictions`

- `prediction_feedback`

- `tickets`

- `users`

- `roles`

- `audit_logs`

---

**13. Central linking via time and production run**

All data must be brought onto the same timeline.

Every record should preferably have these fields:

- `company_id`

- `site_id`

- `line_id`

- `machine_id`

- `production_run_id`

- `material_id`

- `material_batch_id`

- `timestamp`

- `source_id`

Only then can you later say:

This pressure fluctuation occurred with this material, on this machine, during this production run, and later led to this quality issue.

That is the real foundation for AI.

---

**14. Event Normalization Layer**

Because every machine uses different signal names, you need a translation layer.

Example:

`Temp_Zone_1`

`Zone1Temp`

`T_Z1`

`HeatingZone01`

are all mapped to a canonical name:

`extruder.temperature.zone_1`

Likewise:

- `screw_speed`

- `motor_load`

- `melt_pressure`

- `throughput`

- `material_flow`

This prevents each model from working on only one machine.

---

**15. Data quality service**

A dedicated backend service should continuously check:

- Missing values

- Constant values

- Unrealistic jumps

- Time gaps

- Delayed data

- Duplicate records

- Sensor offline

- Unit errors

The service calculates per data source:

- `completeness`

- `freshness`

- `consistency`

- `validity`

- `availability`

From that, the data quality score is derived.

---

**16. Capability Engine**

This is the central logic behind the interface.

The service regularly checks:

- Which data sources are active?

- What data quality is available?

- How much history exists?

- Are there enough events?

- Does a validated model exist?

Then it sets the status of each feature.

Pseudo-code:

```python

def evaluate_feature(feature, company_id):

sources = get_connected_sources(company_id)

missing_required = [

source

for source in feature.required_sources

if source not in sources

]

if missing_required:

return {

"status": "LOCKED",

"missing_sources": missing_required

}

history_days = calculate_history_days(feature, company_id)

if history_days < feature.minimum_history_days:

return {

"status": "COLLECTING_DATA",

"history_days": history_days,

"required_days": feature.minimum_history_days

}

model = get_latest_validated_model(feature.key, company_id)

if model is None:

return {

"status": "TRAINING"

}

return {

"status": "ACTIVE",

"model_id": model.id

}

```

---

**17. API for the Figma frontend**

The home page should preferably call only one central API:

`GET /api/v1/operations-center/overview`

Example response:

```json

{

"plant_status": "PRODUCTION",

"digitalization_progress": 38,

"prediction_readiness": 52,

"connected_machines": 1,

"total_machines": 20,

"data_quality_score": 87,

"active_predictions": 2,

"active_alarms": 1,

"machines": [

{

"id": "extruder_01",

"status": "PRODUCTION",

"integration_score": 72,

"connected_sensors": 21

},

{

"id": "extruder_02",

"status": "NOT_CONNECTED",

"integration_score": 0

}

],

"locked_features": [

{

"feature": "quality_prediction",

"missing_sources": ["quality_data"]

},

{

"feature": "remaining_useful_life",

"missing_sources": ["maintenance_history"]

}

]

}

```

---

**18. Dashboard reaction after a new connection**

When quality data are connected, for example, the following should happen automatically:

- Progress increases

- Prediction readiness increases

- The quality module changes from `LOCKED` to `COLLECTING_DATA`

- The UI shows:

Quality data successfully connected.

Progress: 38 % → 53 %

Prediction readiness: 52 % → 68 %

Still 24 days of data collection until first validation.

That creates exactly the gamification effect you want.

---

**19. Store progress events**

Every change should be logged:

`progress_events`

Example:

```json

{

"event_type": "DATA_SOURCE_CONNECTED",

"source": "quality_data",

"old_progress": 38,

"new_progress": 53,

"old_readiness": 52,

"new_readiness": 68,

"created_at": "2026-07-22T15:15:00Z"

}

```

That way you can later show:

Your production became 15 % more digital this week.

---

**20. What the developer should build first**

The order should be:

**Phase 1:**

Data source registry. Machine registry. Capability Engine. Digitalization progress. Prediction Readiness Score. Locked-feature logic.

**Phase 2:**

Data quality service. Setup wizard. Machine integration status. History calculation.

**Phase 3:**

Real model registration. Validation metrics. Prediction feedback. Model monitoring.

**Phase 4:**

Quality data. Maintenance data. Material batches. Energy data.

---

**The overall principle**

The interface may already present the full vision today. But for every feature, the backend must clearly distinguish:

- What is really connected?

- What is only prepared?

- What is currently collecting data?

- What is rule-based?

- What is simulated?

- What is a real validated ML result?

That way Zitta can already see today what the complete digital production control center will look like. At the same time, the system stays technically honest. Every further connection visibly leads to more features, higher progress, a better data foundation—and later to actually better predictions.

Another instruction:

Tell the team to treat this as a fast demo redesign, not as a complete rebuild.

The goal is to have a presentable ZITTA Production Intelligence demo very quickly. We need a new industrial skin, a clearer production-leader view and realistic demo data. The existing backend and functions should remain as they are wherever possible.

Please work in the following order:

First, create the new visual skin directly on top of the existing frontend. Do not rebuild the complete application. Reuse the current routes, API calls, components and database structures. Only reorganize the dashboard and replace the current visual design.

The first version only needs these areas:

ZITTA Production Intelligence header

Current production status

Connected machines: for example 1 of 20

Digitalization progress

Prediction readiness

Current machine values

Current warnings

Expected risks

Missing data sources

Locked future functions

A graphical production overview showing the Office IT network, Edge Gateway, the currently connected extruder and the machines that are still not connected

The dashboard should immediately show the customer:

What is already connected

What currently works

Which machine is being monitored

Which functions are still limited

Which data source is missing

What additional benefit becomes available after the next connection

For the first customer presentation, the team may use controlled demo data. However, every value must have a clear source status internally:

LIVE

RULE_BASED

DERIVED

SIMULATED

The demo must never present simulated values as validated AI accuracy. Use labels such as:

Prediction Readiness

Current Data Coverage

Estimated Potential

Demo Prediction

Rule-based Warning

For example:

Prediction Readiness: 42%

Potential after connecting quality data: 68%

Potential after connecting maintenance data: 81%

Potential after connecting all machines: 92%

These values can initially come from a simple configuration file instead of a complex backend calculation.

Create one central demo configuration, for example:

{

"totalMachines": 20,

"connectedMachines": 1,

"digitalizationProgress": 28,

"predictionReadiness": 42,

"dataQuality": 86,

"connectedSources": [

"machine_data",

"machine_state",

"live_sensors"

],

"missingSources": [

"quality_data",

"maintenance_history",

"material_batches",

"energy_data"

]

}

The frontend should calculate the visible progress from this configuration. This allows us to adjust the presentation quickly without changing the whole backend.

The locked functions should be driven by a second configuration:

[

{

"name": "Quality Degradation Prediction",

"status": "locked",

"requires": ["quality_data"],

"benefit": "Earlier detection of quality deterioration"

},

{

"name": "Remaining Useful Life",

"status": "locked",

"requires": ["maintenance_history"],

"benefit": "Better maintenance planning"

},

{

"name": "Material Behaviour Analysis",

"status": "partially_available",

"requires": ["material_batches"],

"benefit": "Comparison of material batches"

}

]

When a source is marked as connected, the UI should automatically:

Increase the digitalization progress

Increase prediction readiness

Change the function status

Remove the lock

Show the next implementation step

This does not need a complex AI service for the first demo. A simple capability engine in the frontend or backend is sufficient.

Example logic:

const isAvailable = feature.requires.every(

source => connectedSources.includes(source)

);

For the Figma designer:

Use a serious industrial design.

Dark graphite, steel grey and one strong green accent.

Avoid excessive gradients, playful illustrations and many colors.

The central production map is the most important visual element.

The user should feel that this represents his factory, not a generic software product.

The production overview should show:

Office IT network

Edge Gateway

Connected Extruder 1

19 additional machines in grey

Missing machine network

Missing quality connection

Missing maintenance connection

The connected machine should be visually strong and active. All future machines should remain visible but disabled.

For the developer:

Do not show 20 individual heavy charts on the start page.

Use compact SVG sparklines or simple CSS indicators.

Load detailed charts only after the user opens a machine.

Do not use large videos, heavy 3D models or WebGL.

Avoid constant animations.

Use lazy loading for charts and secondary pages.

Do not poll every sensor separately.

Use one combined dashboard API or one aggregated WebSocket/SSE stream.

Only keep the most recent data points in the browser.

The system must run on the small HP Mini PC. Therefore:

The frontend should remain a normal lightweight React/Vite application.

The backend should provide already aggregated values.

Do not perform ML training on the Mini PC.

Do not calculate large historical datasets in the browser.

Do not continuously query the full SQL database.

Use cached dashboard summaries.

Use a polling interval of approximately 10 to 30 seconds for the demo.

Limit live charts to approximately 50 to 100 visible points.

Load historical data only on demand.

The first demo version should be completed in two short stages.

Stage 1: Visual demo

New ZITTA skin

Production map

One connected machine

Progress indicators

Locked modules

Dummy or rule-based warnings

Responsive desktop layout

Stage 2: Connection to the existing system

Replace demo machine values with current live values

Connect actual machine state

Connect current alarms

Connect existing sensor endpoints

Keep missing modules locked

The team should not wait until every backend function is complete. Build the complete visual vision now, but clearly distinguish between live, simulated and locked content.

The fastest result is therefore:

Keep the existing application.

Build one new Operations Center page.

Use configuration-driven demo data.

Connect only the current live values.

Represent missing functions as locked expansion modules.

Optimize all data processing for the HP Mini PC.

This gives us something impressive to show immediately, while the real data integrations can be added step by step.