# Machine State Service – Technical Documentation

## 1. Overview

The Machine State Service is a FastAPI-based background service that continuously monitors machine sensor values and determines the current machine state based on predefined rules (baselines and mappings).

It works by:

- Fetching live sensor data

- Comparing values against configured thresholds

- Assigning a machine state based on priority logic

- Updating the system when the state changes

## 2. System Architecture

┌──────────────────────────────┐

│        External APIs         │

│------------------------------│

│ /machine-state               │

│ /baselines                   │

│ /default-sensors             │

│ /extruder-latest-values      │

└──────────────┬───────────────┘

│

▼

┌──────────────────────┐

│   Fetch Service      │

│ (get_all_data)       │

└─────────┬────────────┘

│

▼

┌──────────────────────┐

│ State Detection      │

│ (detect_machine_state)

└─────────┬────────────┘

│

▼

┌──────────────────────┐

│ Scheduler Job        │

│ (APScheduler)        │

└─────────┬────────────┘

│

▼

┌──────────────────────┐

│ Update Machine State │

│   (PUT API Call)     │

└──────────────────────┘

## 3. Execution Flow

- Service starts

  - FastAPI triggers startup()

  - Scheduler starts

- Scheduler runs periodically

  - Interval defined by CHECK_INTERVAL

- Each job execution

  - Fetch all required data

  - Detect machine state

  - Compare with previous state

  - If changed → update via API

## 4. Data Sources

The system fetches:

| API | Purpose |
| --- | --- |
| /machine-state/default-machine-states | Defines states & priority |
| /baselines/baseline-maps | Defines rules for each state |
| /default-sensors | Sensor definitions |
| /dashboard/extruder-latest-values | Live sensor values |

## 5. Machine State Detection Logic

### Step 1: Sort by Priority

Machine states are sorted in ascending order of priority:

### Step 2: Iterate Baselines

Each baseline contains mappings for machine states.

For each baseline:

- Loop through priorities

- Find corresponding mapping

- Validate sensor conditions

### Step 3: Sensor Matching Logic

Each machine state has multiple sensor conditions:

IF all sensor conditions match → state is selected

ELSE → move to next state

## 6. Min-Max Value Logic

Each sensor mapping contains:

{

"sensor_id": 1,

"min_value": 10,

"max_value": 50

}

### Evaluation Rule:

min_value ≤ actual_sensor_value ≤ max_value

### Example:

| Sensor | Min | Max | Actual | Result |
| --- | --- | --- | --- | --- |
| Temp | 10 | 50 | 30 | ✅ Pass |
| Speed | 100 | 300 | 350 | ❌ Fail |

👉 If any sensor fails, the entire state is rejected.

## 7. Sensor Mapping

Internal mapping converts sensor IDs to actual keys:

SENSOR_MAP = {

1 → Val_1

2 → Val_6

3 → Val_7

4 → Val_8

5 → Val_9

6 → Val_10

}

This connects:

- Configuration → Live data fields

## 8. Baseline Concept

A baseline is a rule set defining how states should behave.

### Structure:

Baseline

└── Machine State Mapping

└── Sensor Conditions

### Example:

Baseline A:

- PRODUCTION:

Sensor 1 → 10–50

Sensor 2 → 100–300

- LOW_PRODUCTION:

Sensor 1 → 50–70

### How Baselines Work

- Multiple baselines can exist

- System loops through each baseline

- First matching state (by priority) is selected

## 9. Fallback Behavior

If no state matches:

→ System returns last priority state (usually OFF)

This ensures:

- System always returns a valid state

- Avoids null states

## 10. State Change Optimization

The system avoids unnecessary API calls:

IF new_state != PREV_STATE:

→ update API

ELSE:

→ do nothing

This improves:

- Performance

- Network usage

- Stability

## 11. API Update Mechanism

When state changes:

PUT /machine-status

{

"status": "RUNNING"

}

If success:

- State is stored as PREV_STATE

If failure:

- Error is logged

- State is NOT updated internally

## 12. Error Handling

Handled scenarios:

- Missing sensor mapping

- Missing sensor values

- API failures

- Unexpected exceptions

Fallback:

If the API fails, this service does not update the machine state.

{

"machine_state_id": None,

"machine_state_name": "UNKNOWN"

}

## 13. How Users Define Machine States

Users configure states via backend APIs.

### Steps:

#### 1. Define Machine States

{

"id": 1,

"name": "RUNNING",

"priority": 3

}

#### 2. Define Baseline Mapping

{

"machine_state_id": 1,

"mappings": [

{

"sensor_id": 1,

"min_value": 10,

"max_value": 50

}

]

}

### Best Practices

- Keep ranges non-overlapping

- Use priority carefully

- Define OFF as last priority

- Ensure all sensors exist in mapping

## 14. Example Full Flow

Live Data:

Val_1 = 30

Val_6 = 200

Check RUNNING:

✔ Val_1 in range

✔ Val_6 in range

→ State = RUNNING

→ API updated

## 15. Key Advantages

- Real-time monitoring

- Configurable logic (no code changes needed)

- Priority-based safety handling

- Efficient state updates

## 16. Summary

This service acts as a rule engine:

- Inputs → Sensor Data

- Rules → Baselines + Min/Max thresholds

- Output → Machine State

It ensures:

- Accurate detection

- Minimal API load

- Flexible configuration